SUPPLEMENTAL MATERIALS

Supplementary Table S1.  Breath Testing Devices Identifying Numbers and Specifications

| Breath Testing Device | Identifying Numbers | Manufacturer Specifications |
| --- | --- | --- |
| DRIVESAFE Evoc #1 | FCC ID: 2AFL6-I-533 | Sensor: “electrochemical” Claimed accuracy: “+/- 5%@ 0.50 mg/: (100 mg/dL, 0.10 g/L)” |
| DRIVESAFE Evoc #2 | FCC ID: 2AFL6-I-533 |  |
| BACtrack Mobile Pro #1 M5 (model) | SN: GM6-0019, FCC ID: S8B-BTBLE40, IC: 10935A-BTBLE40 | Sensor: “fuel cell” Claimed accuracy: not specified |
| BACtrack Mobile Prog #2 M5 (model) | SN: GM6-0018, FCC ID: S8B-BTBLE40, IC: 10935A-BTBLE40 |  |
| Alcohoot #1 | N/A | Sensor: “fuel cell” |
| Alcohoot #2 | N/A |  |
| BACtrack Vio (Keychain) #1 | 5546096 | Sensor: “Microcheck semiconductor” Claimed accuracy: Not specified |
| BACtrack Vio (Keychain) #2 | 5546001 |  |
| Floome #1 | CAACFLO00002-DPO17003280 | Sensor: “Electrochemical fuel cell” Claimed accuracy: not specified |
| Floome #2 | CAACFLO00002-DPO17003387 |  |
| Drinkmate #1 | N/A | Sensor: not specified Claimed accuracy: “+/- 0.01% BAC at a BAC of 0.02%” |
| Drinkmate #2 | N/A |  |

N/A: Not applicable (no identifying serial or device numbers on device or within package)

The Intoxilyzer 240 is considered by the US Department of Transportation to be an Approved Evidential Breath Testing Device for law enforcement and prosecution of alcohol impaired driving. https://www.transportation.gov/odapc/approved-evidential-breath-testing-devices and https://www.govinfo.gov/content/pkg/FR-2017-11-02/pdf/2017-23869.pdf.

Supplementary Table S2. Study activities at Site Visit

| Time | Procedures * |
| --- | --- |
| 2 hours prior to arrival | - Participants instructed to eat breakfast 2 hours prior to arrival at the lab |
| Lab arrival | - Participants eligibility confirmed, consent completed, weighed, and tested with Intoxilyzer 240 to confirm BrAC 0.00%. |
| 00 - 10 minutes | - Participant is given a priming dose of alcohol containing vodka in fruit juice designed to raise the BrAC to 0.03-0.04 g/dL based on calculations based on weight and gender |
| 10-30 minutes | - Participant rinses with alcohol free mouthwash. |
| 30-40 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro)** |
| 40-50 minutes | - Second dose of alcohol designed to raise BrAC to 0.07-.08’ |
| 50-70 minutes | - Participant rinses with alcohol free mouthwash. |
| 70-80 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro) |
| 80-90 minutes | - Participant receives third dose of alcohol containing vodka in fruit juice designed to raise the BrAC to 1.01-1.02 g/dL |
| 90-110 minutes | - Participant rinses with alcohol free mouthwash. |
| 110-120 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro) - Blood draw to determine Blood Alcohol Content |
| 135 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro |
| 150 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro) |
| 175 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro) |
| 190 minutes | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro) |
| 205 minutes until study completion | - 4 measurements of BrAC (Intoxilyzer, Alcohoot, Drivesafe Evoc, BACtrack Mobile Pro) - Breathalyzer measurements will be obtained until participant is at .02 or lower |

* Note that the above table is for participants 1-10. For participants 11-20, the same format was used but with the second series of breath testing devices: Backtrack Vio, DrinkMate, and Floome.

** Breath testing device measurements were randomized using 4x4 Latin squares within each testing interval. The two commercial test devices were rotated between study visits (e.g., participant #1 used BACTrack Mobile Pro device #1, participant #2 used BACTrack Mobile Pro device #2, participant #3 used BACTrack Mobile Pro device #1, etc.