Ultraviolet radiation exposure in cannabis growing facilities: Supplementary Materials

Maximilian J. Chmielinski1*, Patricia O. Ehrlich1, Martin Cohen1, Tania M. Busch Isaksen1, Christopher D. Simpson1

1Department of Environmental and Occupational Health Sciences, University of Washington, 4225 Roosevelt Way NE Suite 100, Seattle, WA, 98105, USA.

Key Words: Cultivation; horticulture; indoor; marijuana; non-ionizing radiation; UV

Table S1. Summary statistics of measured lamp irradiance grouped by lamp category and ordered by average measured irradiance in W/cm2.

| Type | n | Minimum (eff. W/cm2) | Maximum (eff. W/cm2) | Average (eff. W/cm2) | SD (eff. W/cm2) | Time to Exceed TLV (min) | Percent Above LOD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Germicidal | 2 | 2.34E-04 | 5.85E-04 | 4.09E-04 | 2.48E-04 | 0.12 | 100 |
| Metal Halide | 8 | 3.30E-09 | 1.59E-07 | 6.95E-08 | 7.39E-08 | 720 | 100 |
| HPS | 9 | >1.87E-09 | 2.31E-08 | 6.76E-09 | 7.56E-09 | 7401 | 44 |
| Fluorescent | 7 | >1.87E-09 | 1.65E-08 | 3.96E-09 | 5.53E-09 | 12623 | 14 |
| LED | 14 | >1.87E-09 | 2.64E-09 | 1.98E-09 | 2.79E-10 | 25254 | 14 |

One foot measurement below radiometer limit of detection.

Irradiance at three feet was below radiometer LOD of 2.4x10-8 (eff. W/cm2). Value presented is estimated at three feet using the inverse square law, from measurements made at one foot from the lamp center.