Supplemental Document for Manuscript: The need for better metrics for floor-tile topography: Conventional metrics correlate only modestly with shoe-floor friction

- Data availability for all measurements

All data have been made publicly accessible via DOI for download and supplementary analyses. The measured data for COF and roughness parameters are available in a public repository [1]. The raw data from all surface measurements is also accessible using the contact.engineering web application, with DOIs for each digital surface twin listed in Table S1.

Table S1: List of flooring tiles used, along with DOI for accessing all surface-topography measurements for each tile.

| Manufacturer | Tile Product | Finish | Tile Code | DOI |
| --- | --- | --- | --- | --- |
| Crossville | Graphite | Polished | BPOL | https://doi.org/10.57703/ce-bpczg |
| Crossville | Graphite | Unpolished Sheen | BUPS | https://doi.org/10.57703/ce-cxyn8 |
| Fired Earth | Piemonte | Matte | PPM | https://doi.org/10.57703/ce-wkq6z |
| Fired Earth | Piemonte | Polished | PPP | https://doi.org/10.57703/ce-j5qtr |
| Fired Earth | Piemonte | Satin-Bone | PBS | https://doi.org/10.57703/ce-yhzx9 |
| Fired Earth | Piemonte | Satin-Pearl | PPS | https://doi.org/10.57703/ce-v6t8p |
| Crossville | Roasted Chestnut | Polished | APOL | https://doi.org/10.57703/ce-zcpbh |
| Crossville | Roasted Chestnut | Unpolished Sheen | AUPS | https://doi.org/10.57703/ce-mdqfs |
| Crossville | Venho Verde | Unpolished Sheen | CUPS | https://doi.org/10.57703/ce-8tzq3 |
| Fired Earth | Umbria | Polished | UMBP | https://doi.org/10.57703/ce-g4cym |
| Fired Earth | Umbria | Matte | UMBM | https://doi.org/10.57703/ce-crtwt |
| Fired Earth | Umbria | Satin | UMBS | https://doi.org/10.57703/ce-wbp5n |
| Fired Earth | Lombardia | Polished | LMBP | https://doi.org/10.57703/ce-6qy4a |
| Fired Earth | Lombardia | Matte | LMBM | https://doi.org/10.57703/ce-ynqrq |
| Fired Earth | Lombardia | Satin | LMBS | https://doi.org/10.57703/ce-r5cau |
| Crossville | State of Grace | Unpolished Sheen | FUPS | https://doi.org/10.57703/ce-57etj |
| Crossville | State of Grace | Satin | FSAT | https://doi.org/10.57703/ce-nu46d |
| Crossville | Juno | Unpolished Sheen | EUPS | https://doi.org/10.57703/ce-b3tjf |
| Crossville | Juno | Honed | EHON | https://doi.org/10.57703/ce-4pq7x |
| American Wonder | Asher Haze | Finish 1 | AH1 | https://doi.org/10.57703/ce-9wzs6 |
| American Wonder | Asher Haze | Finish 2 | AH2 | https://doi.org/10.57703/ce-2j7be |
| American Wonder | Venato | Polished | VPOL | https://doi.org/10.57703/ce-mg4cy |
| American Wonder | Polar Style | White | PSW | https://doi.org/10.57703/ce-zaf32 |

- Power spectral density analysis

  - Approach to acquiring power spectral density plots from stylus profilometry data

The power spectral density (PSD, C(q)) is a mathematical tool that transforms roughness profiles from the spatial domain (h(x) where h is the height and x is the spatial position) to the wavevector (large wavevectors are associated with shorter wavelengths), q. Plots of PSD enable visualization of the contributions of different wavevectors to the topography of a surface. Mathematically, the PSD is the Fourier transform h of the autocorrelation function, or the square of the Fourier transform of the height function itself. The power spectral density was computed according to the approach of Ref. [2]:

Cq= L-1|hq|2= L-10Lh(x)e-iqxdx2		Eq. (S1)

where L is the scan length. Sanner et al. [3] investigated how to determine the crossover to q-4 (λ4) in a power spectral density which translates into large wavevector (small-wavelength) cutoff which is different for each surface in Fig. 1. The authors suggested a tip-radius reliability cutoff, where they compared scale-dependent root-mean-square curvature to tip curvature (h”(l)), Eq. (S2). Building on that idea, the authors proposed an additional metric to accurately detect the onset of the tip-radius artifact. The metric was the minimum curvature value at a specific scale ‘l’, where D(l) is a differential operator:

hmin''l= -mink[Dl2Dlx2hxk]				Eq. (S2)

For each tip radius and surface topography, there is a critical length scale below which the stylus profilometry data is unreliable. This critical length scale depends both on the tip radius and the curvature of the measured surface, which implies that rougher surfaces have more tip artifacts than smoother surfaces. This is because a tip that can conform to the valleys of a smoother surface may not be able to conform to the valleys of the rougher surfaces. This is why unpolished tiles may have a larger small-wavelength cutoff than the polished tiles.

  - Power spectral density results

Power spectral density analysis plots are provided for each type of flooring tile. Detailed information for each tile (Section 2.1, Table 1) and detailed information about stylus profilometry measurement methods (Section 2.3) are available in the main text.

Supplemental Figure 1: Power spectral density plots across wavevectors. Note that smaller wave lengths correspond with larger wavevectors.

- Nonparametric statistical analyses

In the nonparametric analyses, positive but weak correlations were observed between each roughness parameter and the COF. Spearman correlation coefficients, rs, were between 0.160 and .719. The rs value (p-value) between COF and each roughness parameter were as follows: Ra was 0.675 (p<0.001), Rq was 0.584 (p=0.003), Rp was 0.719 (p<0.001), and Wa was 0.492 (p=0.017).

- References:

[1] Randolph, A.B., Reifler, K., Chadha, V., Jacobs, T.D.B., Beschorner, K.E.: Data Set for Manuscript: The need for better metrics for floor-tile topography: Conventional metrics correlate only modestly with shoe-floor friction. Place  (2023)

[2] Jacobs, T.D., Junge, T., Pastewka, L. Quantitative characterization of surface topography using spectral analysis. Surface Topography: Metrology and Properties 5:013001 (2017).

[3] Sanner, A., Nöhring, W.G., Thimons, L.A., Jacobs, T.D., Pastewka, L. Scale-dependent roughness parameters for topography analysis. Applied Surface Science Advances 7:100190 (2022).