Supplementary Table 1: Summary of environmental air and surface samples collected in Seattle and Portland

|  | Air Samples |  |  | Surface Samples |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location | n | Meth concs. [µg/m3] | Fentanyl concs. [µg/m3] |  | n | Meth values [ng/cm2] | Fentanyl values [ng/cm2] |
| Seattle: Urban Core | 3 | ND, ND, 0.003 | No airborne fentanyl detected above LOQ in n=15 samples |  | 3 | 0.012, 0.014, 0.015 | No surface fentanyl detected above LOQ in n=14 samples |
| Seattle: Urban Residential | 3 | ND, ND, ND |  |  | 3 | ND, ND, ND |  |
| Seattle: Residential | 3 | ND, ND, 0.006 |  |  | 3 | ND, ND, ND |  |
| Portland: Urban Core | 3 | ND, 0.005, 0.005 |  |  | 2 | ND, 0.015 |  |
| Portland Suburb | 3 | ND, ND, ND |  |  | 3 | ND, ND, ND |  |

ND = Not detected; not detected above the laboratory LOQ of 1 ng (for both fentanyl and methamphetamine). For a four-hour air sample at 2 L/min, this LOQ would equate to a concentration of 0.002 µg/m3. For a 100 cm2 surface sample this LOQ would equate to a surface loading of 0.01 ng/cm2.

Supplementary Table 2: Comparison of highest methamphetamine environmental air and surface samples to air and surface samples collected from transit vehicles

| Highest Environmental Air Methamphetamine: 0.006 µg/m3 |  |
| --- | --- |
| n (%) transit air samples below highest environmental levels: | 1 (1.3%) |
| n (%) transit samples > 10x environmental (>0.06 µg/m3): | 10 (13%) |
| n (%) transit samples > 100x environmental (>0.6 µg/m3): | 4 (5%) |
| Median % difference in buses/trains compared to environmental: | 311% |
| Highest Environmental Surface Methamphetamine: 0.015 ng/cm2 |  |
| n (%) transit surface samples below highest environmental level: | 3 (3%)* |
| n (%) transit samples > 10x environmental (>0.15 ng/cm2): | 71 (71%) |
| n (%) transit samples > 100x environmental (>1.5 ng/cm2): | 18 (15%) |
| Median % difference in buses/trains compared to environmental: | 2256% |

*This includes two surface samples which were below ND (below laboratory LOQ)
Percent differences calculated as: ((transit value – enviro value / (enviro value)) * 100