Supplemental Information

Exposure to Per- and Polyfluoroalkyl Substances (PFAS) in North Carolina Homes: Results from the Indoor PFAS Assessment (IPA) Campaign

Naomi Y. Chang,a Clara M.A. Eichler,a Elaine A. Cohen Hubal,b Jason D. Surratt,a,c Glenn C. Morrison,a Barbara J. Turpina*

aDepartment of Environmental Sciences and Engineering, Gillings School of Global Public Health, University of North Carolina at Chapel Hill, Chapel Hill, NC, USA.

bCenter for Public Health and Environmental Assessment, Office of Research and Development, U.S. Environmental Protection Agency, Research Triangle Park, NC, USA

cDepartment of Chemistry, College of Arts and Sciences, University of North Carolina at Chapel Hill, Chapel Hill, NC, USA

*Corresponding author: bjturpin@email.unc.edu

No. of Pages: 30

No. of Figures: 4

No. of Tables: 17

Table of Contents

Section S1: Targeted Ionic and Neutral Per- and Polyfluoroalkyl Substances (PFAS)	3

Section S2: Sample and Blank Collection Details	6

Tap Water	6

Sample Wipe Collection	6

Surface Wipe Field Blank Collection	6

Mounted Glass Slabs (GS)	7

Dust Collection and Sample Processing	8

Section S3: Quality Assurance and Quality Control	9

Section S4: Exposure Assessment Parameters	14

Section S5. Results	15

Section S6: Exposure Assessment Estimates	20

References	25

# Section S1: Targeted Ionic and Neutral Per- and Polyfluoroalkyl Substances (PFAS)

Table S1. Targeted ionic and neutral PFAS including the CAS#, formula, and paired mass-labelled standard used for quantification. Ionic PFAS were analyzed via ultra-high performance liquid chromatrography-electrospray ionization coupled to tandem mass spectrometry (UHPLC-ESI-MS/MS) operated in the negative mode with multiple reaction monitoring (MRM). Neutral PFAS were analyzed via gas chromatography mass spectrometry in electron ionization mode. Additional details regarding analyses via UHPLC-ESI-MS/MS1,2 and GC-EI-MS3 are provded elsewhere.

| PFAS Acronym | PFAS | CAS# | Formula | Mass-Labelled PFAS Standard |
| --- | --- | --- | --- | --- |
| Ionic PFAS – Analyzed by UHPLC-ESI-MS/MS |  |  |  |  |
| PFBA | Perfluoro-n-butanoic acid | 375-22-4 | C4HF7O2 | 13C4-PFBA |
| PFPeA | Perfluoro-n-pentanoic acid | 2706-90-3 | C5HF9O2 | 13C5-PFPeA |
| PFBS | Perfluorobutane sulfonic acid | 375-73-5 | C4HF9O3S | 13C3-PFBS |
| PFHxA | Perfluoro-n-hexanoic acid | 307-24-4 | C6HF11O2 | 13C5PFHxA |
| GenX | Hexafluoropropylene oxide-dimer acid (HFPO-DA) | 13252-13-6 | C6HF11O3 | 13C3-PFPrOPrA |
| PFPeS | Perfluoropentane sulfonic acid | 2706-91-4 | C5HF11O3S | 13C3-PFBS |
| PFHpA | Perfluoro-n-heptanoic acid | 375-85-9 | C7HF13O2 | 13C4-PFHpA |
| PFHxS | Perfluorohexane sulfonic acid | 355-46-4 | C6HF13O3S | 13C3-PFHxS |
| PFOA | Perfluoro-n-octanoic acid | 335-67-1 | C8HF15O2 | 13C8-PFOA |
| PFHpS | Perfluoroheptane sulfonic acid | 375-92-8 | C7HF15O3S | 13C3-PFHxS |
| PFNA | Perfluoro-n-nonanoic acid | 375-95-1 | C9HF17O2 | 13C9-PFNA |
| PFOS | Perfluorooctane sulfonic acid | 1763-23-1 | C8HF17O3S | 13C8-PFOS |
| PFDA | Perfluoro-n-decanoic acid | 335-76-2 | C10HF19O2 | 13C6-PFDA |
| PFNS | Perfluorononane sulfonic acid | 68259-12-1 | C9HF19O3S | 13C8-PFOS |
| PFUnA | Perfluoro-n-undecanoic acid | 2058-94-8 | C11HF21O2 | 13C7-PFUdA |
| PFDS | Perfluorodecane sulfonic acid | 335-77-3 | C10HF21O3S | 13C8-PFOS |
| PFDoA | Perfluoro-n-dodecanoic acid | 307-55-1 | C12HF23O2 | 13C2-PFDoA |
| PFTrA | Perfluoro-n-tridecanoic acid | 72629-94-8 | C13HF25O2 | 13C2-PFDoA |
| PFDoS | Perfluorododecane sulfonic acid | 79780-39-5 | C12HF25O3S | 13C8-PFOS |
| PFTA | Perfluoro-n-tetradecanoic acid | 376-06-7 | C14HF27O2 | 13C2-PFTeDA |
| PFHxDA | Perfluoro-n-hexadecanoic acid | 67905-19-5 | C16HF31O2 | 13C2-PFTeDA |
| PFODA | Perfluoro-n-octadecanoic acid | 16517-11-6 | C18HF35O2 | 13C2-PFTeDA |
| 6:2 diPAP | Sodium bis(1H,1H,2H,2H-perfluorooctyl) phosphate | 57677-95-9 | C16H8F26O4PNa | 13C4-12C12H8F26O4PNa |
| 8:2 diPAP | Sodium bis(1H,1H,2H,2H-perfluorodecyl) phosphate | 114519-85-6 | C20H8F34O4PNa | 13C4-12C16H8F34O4PNa |
| 6:2 monoPAP | Sodium 1H,1H,2H,2H-perfluorooctyl phosphate | 57678-01-0 | C8H4F13O4PNa2 | 13C2-12C6H4F17O4PNa2 |
| 8:2 monoPAP | Sodium 1H,1H,2H,2H-perfluorodecyl phosphate | 57678-03-2 | C10H4F17O4PNa2 | 13C2-12C8H4F17O4PNa2 |
| Neutral PFAS – Analyzed by GC-EI-MS |  |  |  |  |
| 6:2 FTOH | 2-(Perfluorohexyl)ethanol | 647-42-7 | C8H5F13O | 13C-6:2 FTOH |
| 8:2 FTOH | 2-(Perfluorooctyl)ethanol | 678-39-7 | C10H5F17O | 13C-8:2 FTOH |
| 10:2 FTOH | 2-(Perfluorodecyl)ethanol | 865-86-1 | C12H5F21O | 13C-10:2 FTOH |
| 8:2 FTAC | 2-(Perfluorooctyl)ethyl acrylate | 27905-45-9 | C13H7F17O2 | 13C-8:2 FTOH |
| 10:2 FTAC | 2-(Perfluorodecyl)ethyl acrylate | 17741-60-5 | C15H7F21O2 | 13C-10:2 FTOH |
| MeFOSA | N-Methylperfluorooctane sulfonamide | 31506-32-8 | C9H4F17NO2S | d-MeFOSA, d-EtFOSA |
| EtFOSA | N-Ethylperfluorooctane sulfonamide | 4151-50-2 | C10H6F17NO2S | d-EtFOSA |
| MeFOSE | N-Methyl-N-(2-hydroxyethyl)perfluorooctane sulfonamide | 24448-09-7 | C11H8F17NO3S | d7-MeFOSE |
| EtFOSE | N-Ethyl-N-(2-hydroxyethyl)perfluorooctane sulfonamide | 1691-99-2 | C12H10O3NSF17 | d7-MeFOSE |

# Section S2: Sample and Blank Collection Details

## Tap Water

Tap water samples and blanks were allowed to come to room temperature and then spiked with 1 ng of mass-labeled PFAS standards. Sample volumes were measured in a graduated cylinder, and then 10 mL of methanol was used to rinse the sample bottles before pouring the sample back into the sample bottle. The graduated cylinder was cleaned with methanol and MilliQ water three times each between each sample blank measurement. WAX SPE cartridges were conditioned with 5 mL of methanol and 10 mL of MilliQ water before loading the samples, a slight modification of EPA Method “Improved Method for Extraction and Analysis of Perfluorinated Compounds (PFCs) from Surface Waters and Well Water by Ultra-High Performance Liquid Chromatography (UPLC)-Tandem Mass Spectrometry (MS/MS) – (SOP ID: D-EMMD-PHCB-043-SOP-03).4 All methanol and water were discarded to waste. The SPE cartridges were dried before passing 5 mL of 25 mM sodium acetate buffer solution through them that was also discarded to waste, as described in the EPA Method (D-EMMD-PHCB-043-SOP-03). The sodium acetate buffer solution was created by mixing 800 mL of 25 mM acetic acid, 1.4 mL of glacial acetic (99.7% purity, ThermoFisher, Waltham, MA) acid into 1 L of MilliQ water, with 200 mL of sodium acetate solution (3.4 g sodium acetate (ThermoFisher, Waltham, MA) into 1 L of MilliQ water). Methanol was passed through the cartridge and discarded to waste before 15 mL polypropylene centrifuge tubes were placed in the collection reservoir of the vacuum manifold. Targeted PFAS were eluted with 4 mL of a 0.1% basic methanol solution made by adding 28% ammonium hydroxide (Alfa Aesar, Ward Hill, MA) to methanol. A new solution of basic methanol was made the day of each sample run. Extracts were then evaporated under the gentle flow of nitrogen to ~25 µL. Around 75 µL of MilliQ water was added to the extract to match the intial mobile phase conditions and then samples were analyzed via LC-MS.

## Sample Wipe Collection

At the first visit, each glass slab was cleaned by wiping three times with two methanol wetted Kimwipe in an established pattern (Figure S1). After each surface was wiped, both Kimwipes were placed in 50 mL polypropylene (PP) centrifuge tubes and stored at 4°C in a cooler for transport back to the laboratory. At lab, samples were stored at -20°C until extraction.

1

2

3

Figure S1. Wipe pattern for mounted glass slabs followed a systematic pattern. Each surface was wiped three times with methanol wetted Kimwipes (x2) from left to right, up and down, and then right to left.

## Surface Wipe Field Blank Collection

At each visit where surface wipes were collected, two field blanks each were collected by waving methanol wetted Kimwipes in the air for 2 min.5 Blanks were then placed in 50 mL PP centrifuge tubes and handled and stored the same as the other surface wipe samples.

## Mounted Glass Slabs (GS)

In the lab, four Command strips (20 lb XL, 3M Company, Saint Paul, MN, USA) were adhered to the back of each GS before they were cleaned in the lab with MilliQ water, methanol, and hexane three times each the day prior to deployment in the field. GS were wrapped in pre-baked aluminum foil and stored in a PP bag for storage and transported to the field this way.

On the first visit to each home, except Home 78, clear tempered glass slabs (30.5 cm x 30.5 cm x 0.3 cm; Rice’s Glass, Carrboro, NC, USA) were mounted using Command Strips onto the walls in the main living area in participants’ homes. No slabs were mounted in Home 78 due to their walls being newly painted. To the extent possible, glass slabs were mounted away from direct sunlight to serve as a comparison to the windows.

## Dust Collection and Sample Processing

Table S2. Home characteristics such as the year built, number of occupants, and flooring type in the main living area were recorded in the home survey conducted at the first visit. For each sampling interval, the dust loading (g m-2) was calculated by dividing the mass of dust vacuumed by the area vacuumed. Air change rates (ACHs) were estimated using a CO2 logger. More details regarding ACH estimates are provided in Eichler et al. (2023).3 See also Eichler et al., (submitted).6

| Home ID | Year Built | No. Occupants | Main Living Area Flooring Type | Sampling Interval | Mass of Dust Vacuumed (< 500 µm; g) | Area Vacuumed (m2) | Dust Loading (g m-2) | ACH, estimate; mean ± std. dev. (h-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65 | 1987 | 3 | Hardwood, rugs | t = 0 | 0.42 | 21.9 | 0.019 | 0.25 ± 0.05 |
|  |  |  |  | t = 6 | 0.36 |  | 0.016 |  |
| 18 | 1993 | 2 | Carpet, linoleum | t = 0 | 0.34 | 32.4 | 0.01 | 0.47 ± 0.31 |
|  |  |  |  | t = 6 | 0.55 |  | 0.017 |  |
| 78 | 1945 | 2 | Vinyl, rugs | t = 0 | 1.39 | 12.9 | 0.11 | 0.40 ± 0.25 |
|  |  |  |  | t = 6 | 1.04 |  | 0.081 |  |
| 30 | 1962 | 2 | Hardwood, rugs | t = 0 | 1.79 | 21.8 | 0.082 | 0.44 ± 0.05 |
|  |  |  |  | t = 6 | 4.92 |  | 0.225 |  |
| 82 | 2017 | 1 | Laminate, rugs | t = 0 | 0.43 | 32.8 | 0.013 | 0.77 |
| 50 | 1954 | 2 | Hardwood, rugs | t = 0 | 2.08 | 25.3 | 0.082 | 0.38 ± 0.04 |
|  |  |  |  | t = 6 | 1.95 |  | 0.077 |  |
| 43 | 1920 | 2 | Laminate, rugs | t = 0 | 1.35 | 13.5 | 0.1 | 0.19 ± 0.06 |
|  |  |  |  | t = 6 | 1.38 |  | 0.102 |  |
| 35 | 1920 | 2 | Laminate, rugs | t = 0 | 2.38 | 27.0 | 0.088 | 0.58 ± 0.25 |
|  |  |  |  | t = 6 | 2.76 |  | 0.102 |  |
| 10 | 1985 | 2 | Bamboo, rugs | t = 0 | 1.3 | 27.5 | 0.047 | 0.50 ± 0.15 |
|  |  |  |  | t = 6 | 1.13 |  | 0.041 |  |
| 59 | 2002 | 4 | Hardwood, rugs | t = 0 | 1.72 | 40.9 | 0.042 | 0.24 ± 0.04 |
|  |  |  |  | t = 6 | 1.28 |  | 0.031 |  |
| 01 | 1999 | 1 | Hardwood, rugs | t = 0 | 2.03 | 32.9 | 0.062 | 0.21 ± 0.02 |
|  |  |  |  | t = 6 | 1.64 |  | 0.05 |  |
| Overall |  |  |  |  |  |  |  |  |
| Mean | 1971 | 2.1 | - | - | 1.48 | 26.3 | 0.06 | 0.4 ± 0.12 |

# Section S3: Quality Assurance and Quality Control

Table S3. Analytical Detection Limits (ADLs) were calculated following EPA methodology and repeating the lowest calibration standard (0.2 ng mL-1 or 1 ng mL-1) seven times. The standard deviation () of the seven runs is then multiplied by the Student’s t-value for a single-tailed 99th percentile t statistic. The ADL was converted from extract concentrations (ng mL-1) to surface concentrations (pg of PFAS per cm-2 of surface) using the average of the GS wiped in homes (930.3 cm2).

| PFAS | ADL (ng mL-1) | Dust ADL (ng g-1) | Surface Wipes ADL (pg cm-2) | Tap Water ADL (ng L-1) |
| --- | --- | --- | --- | --- |
| PFBA | 0.03 | 0.02 | 3.2E-03 | 0.003 |
| PFPeA | 0.02 | 0.01 | 2.1E-03 | 0.002 |
| PFHxA | 0.02 | 0.01 | 2.1E-03 | 0.002 |
| PFHpA | 0.03 | 0.02 | 3.2E-03 | 0.003 |
| PFOA | 0.04 | 0.03 | 4.3E-03 | 0.004 |
| PFNA | 0.03 | 0.02 | 3.2E-03 | 0.003 |
| PFDA | 0.02 | 0.01 | 2.1E-03 | 0.002 |
| PFUnA | 0.04 | 0.03 | 4.3E-03 | 0.004 |
| PFDoA | 0.08 | 0.06 | 8.6E-03 | 0.008 |
| PFTrA* | 0.48 | 0.35 | 5.2E-02 | 0.048 |
| PFTA* | 0.3 | 0.22 | 3.2E-02 | 0.03 |
| PFHxDA* | 0.7 | 0.51 | 7.5E-02 | 0.07 |
| PFODA* | 0.51 | 0.37 | 5.5E-02 | 0.051 |
| L-PFBS | 0.04 | 0.03 | 4.3E-03 | 0.004 |
| L-PFPeS | 0.12 | 0.09 | 1.3E-02 | 0.012 |
| L-PFHxS | 0.07 | 0.05 | 7.5E-03 | 0.007 |
| L-PFHpS | 0.1 | 0.07 | 1.1E-02 | 0.01 |
| L-PFOS | 0.06 | 0.04 | 6.4E-03 | 0.006 |
| L-PFNS | 0.04 | 0.03 | 4.3E-03 | 0.004 |
| L-PFDS | 0.12 | 0.09 | 1.3E-02 | 0.012 |
| L-PFDoS* | 0.42 | 0.30 | 4.5E-02 | 0.042 |
| GenX | 0.47 | 0.34 | 5.1E-02 | 0.047 |
| 6:2 monoPAP* | 0.59 | 0.43 | 6.3E-02 | 0.059 |
| 8:2 monoPAP* | 0.7 | 0.51 | 7.5E-02 | 0.07 |
| 6:2 diPAP* | 0.7 | 0.51 | 7.5E-02 | 0.07 |
| 8:2 diPAP* | 0.74 | 0.54 | 8.0E-02 | 0.074 |

* indicates that the ADL was calculated using a 1 ng ml-1 calibration standard

Table S4. Analytical precision for ionic PFAS collected on Kimwipes (surface wipes), settled dust, and tap water is expressed as a pooled coefficient of variation (CV) of duplicate analyses (n = 17 duplicate pairs). For each PFAS compound, analytical precision was calculated by dividing the pooled standard deviation (σpooled) by the mean of all duplicate analyses. For paired data, σpooled = di22n where d is the difference between paired, i, values, and n is the number of pairs.

| PFAS | Analytical Precision (CV) [%] |  |  |
| --- | --- | --- | --- |
|  | Surface Wipes | Dust | Tap Water |
| PFBA | 0.3 | 13.0 | 11.5 |
| PFPeA | 7.5 | 13.6 | 4.2 |
| PFBS | 0.1 | 13.1 | 2.9 |
| PFHxA | 12.5 | 14.9 | 1.5 |
| GenX | 0.1 | 24.0 | 17.5 |
| PFPeS | - | 18.7 | 9.4 |
| PFHpA | 8.8 | 11.8 | 1.6 |
| PFHxS | 20.5 | 13.1 | 8.8 |
| PFOA | 6.0 | 14.5 | 2.4 |
| PFHpS | 1.5 | 21.2 | 26.2 |
| PFNA | 1.9 | 11.4 | 10.0 |
| PFOS | 5.4 | 9.6 | 21.3 |
| PFDA | 4.2 | 21.2 | 10.2 |
| PFNS | - | 18.8 | 6.6 |
| PFUnA | 5.0 | 12.4 | 28.8 |
| PFDS | 4.3 | 12.0 | 19.2 |
| PFDoA | 6.5 | 14.9 | 20.6 |
| PFTrA | 2.0 | 15.0 | 13.1 |
| PFDoS | - | 11.5 | 41.0 |
| PFTA | - | 30.3 | 33.2 |
| PFHxDA | - | 21.3 | 15.8 |
| PFODA | - | 11.2 | 6.7 |
| 6:2 diPAP | 43.2 | 69.8 | -* |
| 8:2 diPAP | 76.4 | 46.4 | -* |
| 6:2 monoPAP | 2.5 | 13.2 | -* |
| 8:2 monoPAP | 37.9 | 11.0 | -* |

*Analytical precision for 6:2 and 8:2 mono- and diPAP s were not calculated for tap water due to poor recovery.

Table S5. Average recovery for dust samples (n = 6) was determined by spiking residential dust sieved to < 500 um with 45 ng of mass-labeled PFAS (i.e., PFAS internal standard). Not all targeted PFAS have matching internal standards (IS). The standard deviation for PFAS marked with “*” were greater than 25% and results should be interpreted carefully.

| PFAS | Mean (%) | SD (%) |
| --- | --- | --- |
| PFBA IS | 73.8 | 12.2 |
| PFPeA IS | 84.3 | 16.9 |
| PFBS IS | 87.3 | 21.6 |
| PFHxA IS | 71.2 | 15.0 |
| GenX IS | 85.0 | 15.8 |
| PFHpA IS | 70.0 | 13.4 |
| PFHxS IS* | 125.4 | 39.8 |
| PFOA IS | 76.3 | 13.0 |
| PFNA IS | 63.1 | 15.4 |
| PFOS IS | 65.4 | 16.0 |
| PFDA IS | 52.2 | 17.8 |
| PFUnA IS | 65.9 | 9.1 |
| PFDoA IS | 71.6 | 15.8 |
| PFTA IS | 68.9 | 16.0 |
| 6:2 diPAP IS | 68.7 | 24.6 |
| 8:2 diPAP IS* | 172.3 | 84.4 |
| 6:2 monoPAP IS* | 96.8 | 34.8 |
| 8:2 monoPAP IS* | 149.6 | 61.2 |

Table S6. Average recovery and storage recovery for tap water samples. Average recovery was determined by spiking 1 L of MilliQ water with 1 ng of PFAS standards. Three storage blanks were also extracted to assess the effect of storage time on PFAS recovery. Three 1 L bottles of MilliQ water were spiked with 1 ng of analyte PFAS and stored at -80°C with the field collected samples in March 2022. The four PAPs (6:2 monoPAP, 8:2 monoPAP, 6:2 diPAP, and 8:2 diPAP) were excluded from analysis due to poor recoveries, which are provided below for reference.

| PFAS | Recovery (%) | Storage Recovery (%) |
| --- | --- | --- |
| PFBA | 85.8 | 57.2 |
| PFPeA | 91.6 | 72.9 |
| PFBS | 96.5 | 79.8 |
| PFHxA | 88.2 | 64.9 |
| GenX | 87.8 | 68.2 |
| PFPeS | 82.0 | 83.1 |
| PFHpA | 64.7 | 83.2 |
| PFHxS | 99.4 | 85.1 |
| PFOA | 89.5 | 69.7 |
| PFHpS | 88.5 | 56.9 |
| PFNA | 85.0 | 60.7 |
| PFOS | 57.2 | 44.4 |
| PFDA | 75.9 | 40.1 |
| PFNS | 56.5 | 10.1 |
| PFUnA | 48.6 | 9.2 |
| PFDS | 45.6 | 3.8 |
| PFDoA | 48.1 | 3.3 |
| PFTrA | 42.8 | 4.3 |
| PFDoS | 48.7 | 30.6 |
| PFTA | 64.3 | 15.1 |
| PFHxDA | 75.8 | 45.8 |
| PFODA | 37.6 | 28.0 |
| 6:2 diPAP | 173.5 | 6.6 |
| 8:2 diPAP | 1065.7 | 77.9 |
| 6:2 monoPAP | 115.9 | 91.3 |
| 8:2 monoPAP | - | 16.3 |

Table S7. Kimwipe extraction efficiencies (N = 8) for the 26 targeted ionic PFAS were calculated by spiking each Kimwipe with 10 μL of 100 ng mL-1 analyte standard. Kimwipes were extracted following the same method for samples and blanks. Recovery was calculated by dividing the calculated concentration from each Kimwipe by the expected concentration (10 ng mL-1).

| Analyte | Recovery (%) | SD (%) |
| --- | --- | --- |
| PFBA | 85.7 | 15.0 |
| PFPeA | 62.5 | 7.1 |
| PFBS | 64.3 | 9.0 |
| PFHxA | 63.0 | 6.8 |
| GenX | 69.7 | 2.6 |
| PFPeS | 67.0 | 11.3 |
| PFHpA | 56.2 | 9.5 |
| PFHxS | 64.3 | 13.6 |
| PFOA | 45.4 | 6.8 |
| PFHpS | 76.6 | 8.1 |
| PFNA | 41.0 | 6.7 |
| PFOS | 37.8 | 5.7 |
| PFDA | 29.0 | 3.7 |
| PFNS | 18.8 | 4.0 |
| PFUnA | 14.9 | 4.2 |
| PFDS | 13.7 | 3.1 |
| PFDoA | 16.2 | 2.5 |
| PFTrA | 6.5 | 1.8 |
| PFDoS | 14.1 | 4.3 |
| PFTA | 22.4 | 1.8 |
| PFHxDA | 53.9 | 7.9 |
| PFODA | 66.4 | 8.5 |
| 6:2 diPAP | 26.2 | 2.0 |
| 8:2 diPAP | 222 | 31.7 |
| 6:2 monoPAP | 38.3 | 8.4 |
| 8:2 monoPAP | 10.1 | 12.6 |

# Section S4: Exposure Assessment Parameters

Table S8. Exposure parameters and equations used to calculate intake. Dust loading was calculated for the homes that participated in the IPA Campaign by dividing the average mass of dust vacuumed by the floor area that was vacuumed.

| Exposure Route | Sample Media | Daily Intake (DI) Equation (ng day-1 kg-1) | Parameters |  |  | Units | References for Parameters |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Adult | 2-yr old |  |  |
|  |  |  | Body Weight (BW) | 70.8 | 13.2 | kg | Table 8-10 EFH Chapter 87 |
| Fine PM & Gas-Phase Inhalation7 | QFFs; PUF-XAD2-PUF | DI = Cair * IR * AF / BW | Inhalation Rate (IR) | 15.7 | 8 | m3 d-1 | Table 6-1 - EFH Chapter 67 |
|  |  |  | Absorption Fraction (AF) | 0.5 | 0.5 | - | East et al., 20218 |
| Air to Skin Dermal Uptake9,10 |  | DI = Cair * kp_g * SAbody * T / BW | Air Concentration (Cair) | Measured |  | ng m-3 | IPA Campaign |
|  |  |  | kp_g - FOSA/FOSE | 0.18 | 0.18 | m h-1 | Kissel et al., 202310 |
|  |  |  | kp_g - PFAAs | 4.4E-7 | 4.4E-7 | m h-1 | Franko et al., 201211 |
|  |  |  | SABody | 1.93 | 0.53 | m2 | Table 7-9; EFH Chapter 77 |
|  |  |  | Duration of Exposure (T) | 12 | 12 | h d-1 | Estimate |
| Water Ingestion7 | Tap Water | DI = Cwater * IR * AF / BW | Water Concentration (Cwater) | Measured |  | ng L-1 | IPA Campaign |
|  |  |  | Ingestion Rate (IR) | 1.277 | 0.245 | L d-1 | Table 3-1 - EFH Chapter 37 |
|  |  |  | Absorption Fraction (AF) | 0.9 | 0.9 | - | East et al., 20218 |
| Dietary Ingestion7 | Literature | DI = IR / BW | Intake Rate (IR) | 250 | 250 | ng d-1 | Tittlemier et al., 200712 |
| Dust Ingestion7 | Settled Dust | DI = Cdust * IR * AF / BW | Dust Concentration (Cdust) | Measured |  | ng g-1 | IPA Campaign |
|  |  |  | Ingestion Rate (IR) | 0.02 | 0.05 | gd-1 | Table 5-1 - EFH Chapter 57 |
|  |  |  | Absorption Fraction (AF) | 0.9 | 0.9 | - | East et al., 20218 |
| Dust Direct Contact Absorption8 |  | DI = Cdust * DL * TC *T *AF / BW | Dust Load (DL) | 0.066 | 0.066 | g m-2 | IPA Campaign |
|  |  |  | Transfer coefficient (TC) | 0.06 | 0.06 | m2 h-1 | Hubal Cohen et al., 200613 |
|  |  |  | Time doing activity (T) | 8 | 10 | h d-1 | Estimate |
|  |  |  | Absorption Fraction (AF) | 0.048 | 0.048 | % | Fasano et al., 200514 |
| Surface Direct Contact Absorption15 | Glass Slabs | DI = CSurface * Ft *%Hand * TC *ET * AF / BW | Surface Film Conc. (CSurface) | Measured |  | ng cm-2 | IPA Campaign |
|  |  |  | % of SASkin that is Hand (%Hand) | 5.2 | 5.7 | % | Table 7-6; EFH Chapter 77 |
|  |  |  | Fraction Transferred (Ft) | 0.08 | 0.08 | - | US EPA 201215 |
|  |  |  | Transfer coefficient (TC) | 600 | 600 | cm2 h-1 | Cohen Hubal et al., 200613 |
|  |  |  | Exposure Time (ET) | 8 | 12 | h d-1 | Estimate |
|  |  |  | Absorption Fraction (AF) | 0.048 | 0.048 | % | Fasano et al., 200514 |
|  |  | DI = CSurface * CR * EV * T * DT * Fhand * AF / BW / 60 | Contact Rate (CR) | - | 302.1 | cm2 event-1 | Calculated (SA_skin * % Hand) |
| Surface Hand to Mouth Ingestion7 |  |  | Event Frequency (EV) | - | 20 | event h-1 | Adults - Nicas and Best 200816; 2-yr old -Table 4-1 EFH Chapter 47 |
|  |  |  | Time doing activity (T) | - | 12 | h d-1 | Estimate |
|  |  |  | Absorption Fraction (AF) | - | 0.9 | - | East et al., 20218 |
|  |  |  | Duration of Activity (DT) | - | 7 | min h-1 | Table 4-1; EFH Chapter 47 |
|  |  |  | Fraction of hand mouthed (Fhand) | - | 0.13 | - |  |
| Ingestion by Mouthing Clothing17 | Clothing | DI = CCloth * Amouth * n * fHand * AF * ExtSaliva / BW | Clothing Concentration (CCloth) | Measured |  | ng cm-2 | IPA Campaign |
|  |  |  | Area of mouth (AMouth) | - | 10 | cm2 | Gennings et al., 201418 |
|  |  |  | Number of times mouthed (n) | - | 124 | d-1 | Morrison et al., 201517 |
|  |  |  | Fraction of surface mass transferred to hand (fhand) | - | 0.058 | - | Morrison et al., 201517 |
|  |  |  | Absorption Fraction (AF) | - | 0.9 | - | East et al., 20218 |
|  |  |  | Extraction by Saliva (ExtSaliva) | - | 0.48 | - | US EPA, 201215 |
|  |  |  | Biotransformation |  |  |  |  |
|  |  |  | 8:2 FTOH to PFOA | 0.5 | 0.5 | % | Himmelstein et al., 201219 |

EFH refers to the EPA Exposure Factors Handbok (EFH).

# Section S5. Results

Table S9. Summary statistics for tap water samples, which were mean field blank-subtracted and corrected for recoveries. PFAS concentrations noted with “n.d.” indicate a non-detect and values with a “<” in front indicate that the PFAS was detected at concentrations below the MDL.

| PFAS | Mean (ng L-1) | Median (ng L-1) | Min (ng L-1) | Max (ng L-1) | DF (%) | MDL (ng L-1) |
| --- | --- | --- | --- | --- | --- | --- |
| GenX | < 0.047 | n.d. | n.d. | 0.06 | 20 | 0.047 |
| PFBA | 1.53 | 1.31 | < 0.694 | 2.93 | 100 | 0.694 |
| PFBS | 2.72 | 2.66 | < 0.715 | 4.80 | 100 | 0.715 |
| PFDA | 0.11 | 0.06 | n.d. | 0.26 | 90 | 0.002 |
| PFDoA | < 0.048 | n.d. | n.d. | < 0.048 | 30 | 0.048 |
| PFDoS | n.d. | n.d. | n.d. | n.d. | n.d. | 0.042 |
| PFDS | n.d. | n.d. | n.d. | n.d. | n.d. | 0.012 |
| PFHpA | 1.57 | 1.65 | < 0.003 | 3.58 | 100 | 0.003 |
| PFHpS | 0.08 | 0.07 | 0.07 | 0.14 | 100 | 0.068 |
| PFHxA | 1.41 | 0.62 | n.d. | 4.29 | 90 | 0.004 |
| PFHxDA | n.d. | n.d. | n.d. | n.d. | n.d. | 0.051 |
| PFHxS | 0.32 | 0.23 | n.d. | 0.82 | 90 | 0.181 |
| PFNA | 0.15 | 0.18 | n.d. | 0.26 | 90 | 0.011 |
| PFNS | n.d. | n.d. | n.d. | n.d. | n.d. | 0.004 |
| PFOA | 3.60 | 3.76 | < 0.075 | 8.19 | 100 | 0.075 |
| PFODA | n.d. | n.d. | n.d. | n.d. | n.d. | 0.004 |
| PFOS | 2.95 | 2.75 | n.d. | 6.51 | 80 | 0.008 |
| PFPeA | 1.26 | 0.11 | n.d. | 6.51 | 60 | 0.002 |
| PFPeS | 0.18 | n.d. | n.d. | 0.99 | 20 | 0.007 |
| PFTA | 0.08 | 0.07 | n.d. | 0.15 | 90 | 0.07 |
| PFTrA | n.d. | n.d. | n.d. | n.d. | n.d. | 0.03 |
| PFUnA | 0.02 | 0.02 | n.d. | 0.07 | 60 | 0.008 |

Table S10. Summary statistics, method detection limit (MDL), mean field blank, and detection frequencies (DF) for mean field blank subtracted and recovery corrected targeted ionic PFAS on 6-month glass slab wipes. PFAS concentrations noted with “n.d.” indicate a non-detect and values with a “<” in front indicate that the PFAS was detected at concentrations below the MDL.

| PFAS | Mean | Median | Min | Max | MDL | Mean Field Blank | DF |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (pg cm-2) |  |  |  |  |  | % |
| PFBA | 0.63 | <0.48 | n.d. | 2.0 | 0.48 | 8.9E-02 | 78 |
| PFPeA | 2.3E-02 | <2.1E-3 | n.d. | 9.8E-02 | 2.1E-03 | n.d. | 44 |
| PFBS | <4.3E-3 | <4.3E-3 | n.d. | 2.4E-02 | 4.3E-03 | 1.3E-04 | 11 |
| PFHxA | 0.49 | 0.14 | 2.70E-02 | 2.6 | 1.8E-02 | 3.9E-03 | 100 |
| GenX | <5.1E-2 | <5.1E-2 | n.d. | <5.1E-02 | 5.1E-02 | 3.6E-03 | n.d. |
| PFPeS | n.d. | n.d. | n.d. | n.d. | 1.3E-02 | n.d. | n.d. |
| PFHpA | 0.25 | 0.15 | 1.90E-02 | 0.94 | 6.5E-03 | 1.3E-03 | 100 |
| PFHxS | n.d. | n.d. | n.d. | n.d. | 0.59 | 3.9E-03 | n.d. |
| PFOA | 0.78 | 0.63 | 3.10E-02 | 1.7 | 1.6E-02 | 2.0E-03 | 100 |
| PFHpS | 9.3E-02 | <7.1E-2 | <7.1E-2 | 0.25 | 7.1E-02 | 1.1E-02 | 100 |
| PFNA | 0.15 | 0.14 | n.d. | 0.45 | 6.6E-03 | n.d. | 67 |
| PFOS | 0.14 | 0.15 | n.d. | 0.26 | 5.6E-02 | 1.1E-02 | 89 |
| PFDA | 0.14 | 0.15 | 1.90E-02 | 0.37 | 7.1E-03 | 1.3E-03 | 100 |
| PFNS | n.d. | n.d. | n.d. | n.d. | 6.7E-03 | n.d. | n.d. |
| PFUnA | 2.0E-02 | <7.8E-3 | n.d. | 5.5E-02 | 7.8E-03 | 6.6E-04 | 67 |
| PFDS | n.d. | n.d. | n.d. | n.d. | 2.2E-02 | 6.2E-03 | n.d. |
| PFDoA | <6.3E-2 | <6.3E-2 | n.d. | 0.12 | 6.3E-02 | 9.8E-04 | 56 |
| PFTrA | n.d. | n.d. | n.d. | n.d. | 5.2E-2 | 3.3E-05 | n.d. |
| PFDoS | n.d. | n.d. | n.d. | n.d. | 4.5E-02 | n.d. | n.d. |
| PFTA | <5.6E-2 | <5.6E-2 | n.d. | 0.13 | 5.6E-02 | 4.6E-03 | 22 |
| PFHxDA | n.d. | n.d. | n.d. | n.d. | 8.9E-02 | 4.5E-03 | n.d. |
| PFODA | n.d. | n.d. | n.d. | n.d. | 5.5E-2 | n.d. | n.d. |
| 6:2 diPAP | 1.7 | <0.85 | n.d. | 9.4 | 0.85 | 8.7E-02 | 67 |
| 8:2 diPAP | <0.14 | <0.14 | n.d. | <0.14 | 0.14 | 3.9E-03 | 67 |
| 6:2 monoPAP | 0.1 | <6.3E-2 | n.d. | 0.71 | 6.3E-02 | 3.3E-03 | 22 |
| 8:2 monoPAP | 0.14 | <7.5E-2 | n.d. | 1.0 | 7.5E-2 | 3.5E-03 | 33 |

Figure S2. PAP profiles and concentrations on glass slabs collected at 6-months.

Table S11. Overall summary statistics for recovery corrected dust samples. Values are median blank subtracted and the MDL was taken as the 95th percentile concentration following UC Davis’ IMPROVE methodology.20 PFAS concentrations noted with “n.d.” indicate a non-detect and values with a “<” in front indicate that the PFAS was detected at concentrations below the MDL.

| PFAS | Mean (ng g-1) | Median (ng g-1) | Min (ng g-1) | Max (ng g-1) | F.Blk (ng g-1) | DF (%) | MDL (ng g-1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PFBA | 28.0 | 9.2 | n.d. | 284 | n.d. | 76 | 4.6 |
| PFPeA | 0.6 | n.d. | n.d. | 3.6 | n.d. | 43 | 0.25 |
| PFBS | <4.8 | <4.8 | n.d. | 22.5 | 3.4 | 10 | 4.8 |
| PFHxA | 9.0 | 7.2 | <3.6 | 30.3 | n.d. | 62 | 3.6 |
| GenX | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.39 |
| PFPeS | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.06 |
| PFHpA | 1.6 | <1.1 | n.d. | 9.7 | n.d. | 38 | 1.1 |
| PFHxS | 1.4 | n.d. | n.d. | 19.9 | n.d. | 10 | 0.06 |
| PFOA | 8.0 | 8.3 | n.d. | 31.4 | n.d. | 81 | 0.04 |
| PFHpS | <0.66 | n.d. | n.d. | 1.6 | n.d. | 5 | 0.66 |
| PFNA | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.03 |
| PFOS | 129 | 80.8 | <37.1 | 402 | 0.9 | 76 | 37.1 |
| PFDA | 0.5 | n.d. | n.d. | 2.6 | n.d. | 38 | 0.03 |
| PFNS | 0.1 | n.d. | n.d. | 1.7 | n.d. | 5 | 0.04 |
| PFUnA | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.09 |
| PFDS | <11.1 | n.d. | n.d. | 76.0 | n.d. | 10 | 11.1 |
| PFDoA | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.48 |
| PFTrA | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.64 |
| PFDoS | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.46 |
| PFTA | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.73 |
| PFHxDA | <0.53 | n.d. | n.d. | 8.4 | n.d. | 5 | 0.53 |
| PFODA | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | 0.04 |
| 6:2 diPAP | <293 | <293 | n.d. | 735 | 0.5 | 24 | 293 |
| 8:2 diPAP | <79.3 | <79.3 | n.d. | 112 | 1.7 | 10 | 79.3 |
| 6:2 monoPAP | <69 | <69 | n.d. | 201 | n.d. | 10 | 69 |
| 8:2 monoPAP | 1.0 | n.d. | n.d. | 8.0 | n.d. | 19 | 0.33 |

Table S12. Ionic PFAS concentrations in dust from studies in indoor environments in the US, Canada, Finland, and Ireland. PFAS marked with “-“ indicate that the compound was not targeted. Values provided with “<” were detected at concentrations below quantification limits for other studies and below the MDL for IPA Campaign homes. *Eriksson and Kärrman (2015)21 reported “n.a.” values for PFAS with detection frequencies below 50%.

| Study | Year | Location | Size (μm) |  | PFOA | PFOS | PFBS | PFNA | PFHxA | PFHpA | PFHxS | PFDA | 6:2 diPAP | 8:2 diPAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPA Campaign | 2021 - 2022 | Homes (n = 11); North Carolina | 500 | Median (ng g-1) | 8.3 | 80.8 | <4.8 | n.d. | 7.2 | <1.1 | n.d. | n.d. | <293 | <79.3 |
| Zheng et al., 201922 | Not reported | Daycares (n = 20); WA, IN | 500 | Median (ng g-1) | 2.0 | 1.2 |  | 1.7 | 1.4 | 0.61 | 0.25 | 0.59 | - | - |
| Harrad et al., 201923 | 2016 - 2017 | Homes (n = 34); Ireland | 500 | Median (ng g-1) | 0.42 | 0.96 | 10 | <0.05 | - | - | <0.1 | - | - | - |
| Schildroth et al., 202224 | 2016 | University spaces (n = 43); New England, USA | 150 | Median (ng g-1) | 100 | 41 | - | 79 | 38 | <LOQ | <LOQ | 53 | - | - |
| Hall et al., 202025 | 2014 - 2016 | Homes (n = 184); North Carolina | 500 | Median (ng g-1) | 7.9 | 4 .4 | - | 3 | 9 | 9 | 2 | 6 | 113 | <0.48 |
| Winkens et al., 201826 | 2014 - 2016 | Homes (n = 65); Kuopio, Finland | 500 | Median (ng g-1) | 741 | 1890 | 20.5 | 85.1 | 336 | 374 | 1430 | 63.8 | 687 | 225 |
| Weiss et al., 202132 | 2013 - 2014 | Homes (n = 17); Stockholm and Uppsala regions, Sweden | 1000 | Median (ng g-1) | 9.0 | 13 | < LOQ | 3.4 | 6..4 | 2.2 | < LOQ | 3.2 | 65 | 49 |
| Eriksson and Kärrman (2015)21 | 2013 – 2014; Spain - 2009 | Homes; Australia (n = 10) | 150 | Median (ng g-1) | 13.5 | 9.72 | 1.29 | 2.97 | 4.77 | 2.18 | 4.32 | 5.18 | 83.5 | 48.5 |
|  |  | Homes; Canada (n = 10) |  |  | 21 | 7.29 | 3.44 | 13.5 | 7.44 | 4.75 | 3.75 | 7.07 | 164 | 27.1 |
|  |  | Homes; Faroe Island (n = 10) |  |  | 15.3 | 5.75 | 1.88 | 2.39 | 7.96 | 2.26 | 0.37 | 4.81 | 106 | 46.9 |
|  |  | Homes; Greece (n = 7) |  |  | 12.8 | 7.21 | 1.2 | 3.55 | 3.85 | n.a.* | 0.41 | 5.4 | 5.26 | 3.6 |
|  |  | Homes; Japan (n = 5) |  |  | 25.5 | 3.91 | 1.33 | 35.8 | 12.0 | 6.54 | 0.76 | 10.5 | 119 | 80.8 |
|  |  | Homes; Nepal (n = 10) |  |  | n.a.* | n.a.* | n.a.* | n.a.* | n.a.* | n.a.* | n.a.* | n.a.* | 1.16 | 0.72 |
|  |  | Homes; Spain (n = 10) |  |  | 8.81 | 5.29 | 1.44 | 1.72 | 3.39 | 0.90 | 0.39 | 3.72 | 2.08 | 5.96 |
|  |  | Homes; Sweden (n = 10) |  |  | 14.4 | 2.77 | 1.24 | 1.96 | 7.05 | 2.61 | 0.15 | 3.72 | 15.4 | 9.53 |
| Fraser et al., 201327 | 2009 | Offices (n = 31); MA | 500 | GM (ng g-1) | 32 | 14.6 | - | 63 | 10.8 | 27.6 | - | 46.5 | - | - |
|  |  | Homes (n = 30); MA |  |  | 23.7 | 26.9 | - | 10.9 | 8.65 | 12 | - | - | - | - |
|  |  | Cars (n = 13); MA |  |  | 11.4 | 15.8 | - | 14.7 | 5.92 | 8.48 | - | 8.4 | - | - |
| Goosey and Harrad, 201128 | 2007 - 2009 | Homes (n = 10); CO | 500 | Median (ng g-1) | 240 | 310 | - | - | - | - | 240 | - | - | - |
| Knobeloch et al., 201229 | 2008 | Homes (n = 39); WI | 1000 | Median (ng g-1) | 44 | 47 | 1.8 | 12 | 0 | 17 | 16 | 5.7 | - | - |
| De Silva et al., 201230 | 2007 - 2008 | Homes (n = 102); Vancouver, Canada | 150 | Median (ng g-1) | - | - | - | - | - | - | - | - | 460 | 535 |
| Strynar and Lindstrom, 200831 | 2000-2001 | Homes (n = 102) and daycare centers (n = 10); NC and OH | 150 | Median (ng g-1) | 142 | 201 | 9.11 | 142 | 54.2 | 50.2 | 45.5 | 6.65 | - | - |

# Section S6: Exposure Assessment Estimates

Table S13. Summary statistics for air, water, dust, glass slab, and clothing neutral and/or ionic PFAS concentrations used in estimating exposure for adults and children. Note that ‘-‘ indicates the environmental media was not sampled for that specific PFAS and ‘n.d.’ indicates it was not detected.

| Statistic | PFAS | Air (ng m-3) | Water (ng L-1) | Dust (ng g-1) | GS (ng cm-2) | Clothing (ng cm-2) |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | Total PFAS | 27.8 | 16.0 | 824.5 | 4.2E-03 | 0.25 |
| Median |  | 18.5 | 14.3 | 732.4 | 2.7E-03 | 0.13 |
| Min |  | 7.52 | 1.4 | 303.4 | 4.1E-04 | 3.95E-02 |
| Max |  | 108 | 34.1 | 1638.7 | 1.7E-02 | 0.92 |
| Mean | Total Ionic PFAS | 2.3E-02 | 16.0 | 549.8 | 4.2E-03 | - |
| Median |  | 1.6E-02 | 14.3 | 496.5 | 2.7E-03 | - |
| Min |  | 1.3E-03 | 1.4 | 201.7 | 4.1E-04 | - |
| Max |  | 1.4E-01 | 34.1 | 1036.2 | 1.7E-02 | - |
| Mean | Total Neutral PFAS | 27.8 | - | 274.8 | - | 0.25 |
| Median |  | 18.5 | - | 235.9 | - | 0.13 |
| Min |  | 7.52 | - | 101.6 | - | 3.95E-02 |
| Max |  | 108 | - | 602.4 | - | 0.92 |
| Mean | Total PAPs | 3.6E-03 | - | 361.2 | 1.8E-03 | - |
| Median |  | 1.7E-03 | - | 311.9 | 4.8E-04 | - |
| Min |  | n.d. | - | 104.9 | n.d. | - |
| Max |  | 2.4E-02 | - | 991.2 | 1.1E-02 | - |
| Mean | Total PFCAs | 1.6E-02 | 9.7 | 48.1 | 2.3E-03 | - |
| Median |  | 9.2E-03 | 8.6 | 30.1 | 1.8E-03 | - |
| Min |  | n.d. | 0.7 | 2.5 | 1.2E-04 | - |
| Max |  | 0.12 | 22.0 | 346.5 | 7.9E-03 | - |
| Mean | Total PFSAs | 1.8E-03 | 6.3 | 140.5 | 1.9E-04 | - |
| Median |  | 8.7E-04 | 5.7 | 105.6 | 1.9E-04 | - |
| Min |  | n.d. | 0.7 | 29.7 | 5.4E-05 | - |
| Max |  | 6.3E-03 | 12.1 | 405.8 | 3.9E-04 | - |
| Mean | Total FTOHs | 26.7 | - | 207.4 | - | 0.19 |
| Median |  | 17.2 | - | 173.0 | - | 0.11 |
| Min |  | 5.76 | - | 74.7 | - | 2.3E-03 |
| Max |  | 107 | - | 529.8 | - | 0.84 |
| Mean | Total FOSAs/FOSEs | 0.78 | - | 64.2 | - | 6.6E-02 |
| Median |  | 0.71 | - | 46.3 | - | 4.5E-02 |
| Min |  | 0.2 | - | n.d. | - | 2.5E-03 |
| Max |  | 1.26 | - | 208.2 | - | 0.22 |

Table S14. Daily intake rates (ng kg-1 d-1) for each exposure route and total exposure for inhalation, ingestion, and dermal exposure routes as well as percent contribution of each route for a simulated adult. Note that ‘-‘ indicates the environmental media was not sampled for that specific PFAS and ‘n.d.’ indicates it was not detected.

| Exposure (ng kg-1 d-1) |  |  |  |  |  |  |  |  | Percent Contribution (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inhalation | Ingestion - Water | Ingestion - Dust | Dermal - Dust | Dermal - Surface | Air to Skin | ΣExposure | Inhalation | Ingestion Water | Ingestion Dust | Dermal Dust/Surfaces |
| Mean | Total PFAS | 3.1 | 0.26 | 0.21 | 8.9E-05 | 5.7E-7 | 5.3E-3 | 3.56 | 86.7 | 7.3 | 5.9 | 1.5E-01 |
| Median |  | 2.1 | 0.23 | 0.19 | 7.9E-05 | 3.6E-7 | 4.8E-3 | 2.47 | 82.9 | 9.4 | 7.5 | 2.0E-01 |
| Min |  | 0.83 | 0.02 | 7.7E-02 | 3.3E-05 | 5.5E-8 | 1.3E-3 | 0.94 | 89.1 | 2.5 | 8.2 | 1.5E-01 |
| Max |  | 12.0 | 0.55 | 0.42 | 1.8E-04 | 2.2E-6 | 8.5E-3 | 12.9 | 92.4 | 4.3 | 3.2 | < 0.05 |
| Mean | Total Ionic PFAS | 2.5E-3 | 0.26 | 0.14 | 5.9E-05 | 5.7E-7 | - | 0.4 | 0.6 | 64.6 | 34.8 | < 0.05 |
| Median |  | 1.8E-3 | 0.23 | 0.13 | 5.3E-05 | 3.6E-7 | - | 0.36 | 0.5 | 64.4 | 35.1 | < 0.05 |
| Min |  | 1.4E-4 | 0.02 | 5.1E-02 | 2.2E-05 | 5.5E-8 | - | 7.4E-02 | 0.2 | 31.0 | 68.8 | < 0.05 |
| Max |  | 1.5E-2 | 0.55 | 0.26 | 1.1E-04 | 2.2E-6 | - | 0.83 | 1.8 | 66.5 | 31.6 | < 0.05 |
| Mean | Total Neutral PFAS | 3.1 | - | 7.0E-02 | 3.0E-05 | - | 5.3E-3 | 3.2 | 97.6 | - | 2.2 | 1.7E-01 |
| Median |  | 2.0 | - | 6.0E-02 | 2.5E-05 | - | 4.8E-3 | 2.1 | 96.9 | - | 2.8 | 2.3E-01 |
| Min |  | 0.83 | - | 2.6E-02 | 1.1E-05 | - | 1.3E-3 | 0.86 | 96.8 | - | 3.0 | 1.6E-01 |
| Max |  | 12.0 | - | 0.15 | 6.5E-05 | - | 8.5E-3 | 12.1 | 98.7 | - | 1.3 | < 0.05 |
| Mean | Total PAPs | 4.E-4 | - | 9.2E-02 | 3.9E-05 | 2.4E-7 | - | 9.23E-02 | 0.4 | - | 99.5 | < 0.05 |
| Median |  | 1.8E-4 | - | 7.9E-02 | 3.3E-05 | 6.5E-8 | - | 7.95E-02 | 0.2 | - | 99.7 | < 0.05 |
| Min |  | n.d. | - | 2.7E-02 | 1.1E-05 | n.d. | - | 2.67E-02 | n.d. | - | 100 | < 0.05 |
| Max |  | 2.7E-3 | - | 0.25 | 1.1E-04 | 1.4E-6 | - | 0.26 | 1.1 | - | 98.9 | < 0.05 |
| Mean | Total PFCAs | 1.8E-3 | 0.16 | 1.2E-02 | 5.2E-06 | 3.0E-7 | - | 0.17 | 1.0 | 91.9 | 7.1 | < 0.05 |
| Median |  | 1.E-3 | 0.14 | 7.6E-03 | 3.2E-06 | 2.4E-7 | - | 0.15 | 0.7 | 94.2 | 5.2 | < 0.05 |
| Min |  | n.d. | 0.01 | 6.5E-04 | 2.7E-07 | 1.6E-8 | - | 1.26E-02 | n.d. | 94.9 | 5.1 | < 0.05 |
| Max |  | 1.3E-2 | 0.36 | 8.8E-02 | 3.7E-05 | 1.1E-6 | - | 0.46 | 2.9 | 77.9 | 19.2 | < 0.05 |
| Mean | Total PFSAs | 1.9E-4 | 0.10 | 3.6E-02 | 1.5E-05 | 2.5E-8 | - | 0.14 | 0.1 | 73.9 | 25.9 | < 0.05 |
| Median |  | 9.7E-5 | 0.09 | 2.7E-02 | 1.1E-05 | 2.6E-8 | - | 0.12 | 0.1 | 77.3 | 22.6 | < 0.05 |
| Min |  | n.d. | 0.01 | 7.5E-03 | 3.2E-06 | 7.3E-9 | - | 1.87E-02 | n.d. | 59.6 | 40.3 | < 0.05 |
| Max |  | 7.0E-4 | 0.20 | 0.1 | 4.4E-05 | 5.3E-8 | - | 0.3 | 0.2 | 65.4 | 34.4 | < 0.05 |
| Mean | Total FTOHs | 3.0 | - | 5.3E-02 | 2.2E-05 | - | - | 3.01 | 98.2 | - | 1.8 | < 0.05 |
| Median |  | 1.9 | - | 4.4E-02 | 1.9E-05 | - | - | 1.95 | 97.7 | - | 2.3 | < 0.05 |
| Min |  | 0.64 | - | 1.9E-02 | 8.0E-06 | - | - | 0.66 | 97.1 | - | 2.9 | < 0.05 |
| Max |  | 12.0 | - | 0.13 | 5.7E-05 | - | - | 12.0 | 98.9 | - | 1.1 | < 0.05 |
| Mean | Total FOSAs/FOSEs | 8.7E-2 | - | 1.6E-02 | 6.9E-06 | - | 5.3E-3 | 0.11 | 80.0 | - | 15.0 | 4.9 |
| Median |  | 7.9E-2 | - | 1.2E-02 | 5.0E-06 | - | 4.8E-3 | 9.53E-02 | 82.6 | - | 12.4 | 5.1 |
| Min |  | 2.2E-2 | - | n.d. | n.d. | - | 1.3E-3 | 2.33E-02 | 94.2 | - | n.d. | 5.8 |
| Max |  | 0.14 | - | 5.3E-02 | 2.2E-05 | - | 8.5E-3 | 0.2 | 69.4 | - | 26.4 | 4.3 |

Table S15. Daily intake rates (ng kg-1 d-1) for each exposure route and total exposure for inhalation, ingestion, and dermal exposure routes as well as percent contribution of each route for a simulated 2-year old child. Note that ‘-‘ indicates the environmental media was not sampled for that specific PFAS and ‘n.d.’ indicates it was not detected.

|  |  | Exposure (ng kg-1 d-1) |  |  |  |  |  |  |  |  |  | Percent Contribution (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inhalation | Ingestion Water | Ingestion Dust | Dermal Dust | Dermal Surfaces | Ingestion Surfaces | Mouthing |  | Air to Skin | ΣExposure | Inhalation | Ingestion Water | Ingestion Dust/Surfaces | Mouthing | Dermal |
| Mean | Total PFAS | 8.4 | 0.27 | 2.81 | 9.5E-04 | 5.0E-6 | 0.32 | 0.59 | 8.5E-4 |  | 12.4 | 67.9 | 2.2 | 25.2 | 4.8 | < 0.05 |
| Median |  | 5.6 | 0.24 | 2.5 | 8.4E-04 | 3.2E-6 | 0.2 | 0.32 | 5.8E-4 |  | 8.9 | 63.3 | 2.7 | 30.5 | 3.6 | < 0.05 |
| Min |  | 2.3 | 0.02 | 1.03 | 3.5E-04 | 4.9E-7 | 3.1E-2 | 0.09 | 3.2E-5 |  | 3.5 | 65.9 | 0.7 | 30.8 | 2.7 | < 0.05 |
| Max |  | 32.6 | 0.57 | 5.59 | 1.9E-03 | 2.0E-5 | 1.24 | 2.16 | 2.9E-3 |  | 42.2 | 77.3 | 1.4 | 16.2 | 5.1 | < 0.05 |
| Mean | Total Ionic PFAS | 6.9E-3 | 0.27 | 1.87 | 6.3E-04 | 5.0E-6 | 0.32 | - | - |  | 2.5 | 0.3 | 10.8 | 88.9 | - | < 0.05 |
| Median |  | 4.8E-3 | 0.24 | 1.69 | 5.7E-04 | 3.2E-6 | 0.2 | - | - |  | 2.1 | 0.2 | 11.1 | 88.6 | - | < 0.05 |
| Min |  | 3.8E-4 | 0.02 | 0.69 | 2.3E-04 | 4.9E-7 | 3.6E-2 | - | - |  | 0.7 | < 0.05 | 3.2 | 96.8 | - | < 0.05 |
| Max |  | 4.1E-2 | 0.57 | 3.53 | 1.2E-03 | 2.0E-5 | 1.24 | - | - |  | 5.4 | 0.8 | 10.6 | 88.6 | - | < 0.05 |
| Mean | Total Neutral PFAS | 8.43 | - | 0.94 | 3.2E-04 | - | - | 0.59 | 8.5E-4 |  | 10.0 | 84.6 | - | 9.4 | 6.0 | < 0.05 |
| Median |  | 5.6 | - | 0.8 | 2.7E-04 | - | - | 0.32 | 5.8E-4 |  | 6.7 | 83.3 | - | 12.0 | 4.7 | < 0.05 |
| Min |  | 2.28 | - | 0.35 | 1.2E-04 | - | - | 0.09 | 3.2E-5 |  | 2.7 | 83.8 | - | 12.7 | 3.4 | < 0.05 |
| Max |  | 32.6 | - | 2.05 | 6.9E-04 | - | - | 2.16 | 2.9E-3 |  | 36.8 | 88.6 | - | 5.6 | 5.9 | < 0.05 |
| Mean | Total PAPs | 1.1E-3 | - | 1.23 | 4.2E-04 | 2.1E-6 | 0.13 | - | - |  | 1.4 | 0.1 | - | 99.9 | - | < 0.05 |
| Median |  | 5.3E-4 | - | 1.06 | 3.6E-04 | 5.8E-7 | 3.6E-2 | - | - |  | 1.1 | 4.6E-02 | - | 99.9 | - | < 0.05 |
| Min |  | n.d. | - | 0.36 | 1.2E-04 | n.d. | n.d. | - | - |  | 0.4 | n.d. | - | 100.0 | - | < 0.05 |
| Max |  | 7.4E-3 | - | 3.38 | 1.1E-03 | 1.3E05 | 0.79 | - | - |  | 4.2 | 0.2 | - | 99.8 | - | < 0.05 |
| Mean | Total PFCAs | 4.8E-3 | 0.16 | 0.16 | 5.6E-05 | 2.7E-6 | 0.17 | - | - |  | 0.5 | 1.0 | 32.5 | 66.5 | - | < 0.05 |
| Median |  | 2.8E-3 | 0.14 | 0.1 | 3.5E-05 | 2.1E-6 | 0.13 | - | - |  | 0.4 | 0.7 | 37.7 | 61.6 | - | < 0.05 |
| Min |  | n.d. | 0.01 | 8.68E-03 | 2.9E-06 | 1.4E-7 | 9.0E-3 | - | - |  | 3.0E-2 | n.d. | 41.1 | 58.9 | - | < 0.05 |
| Max |  | 3.6E-2 | 0.37 | 1.18 | 4.0E-04 | 9.4E-6 | 0.59 | - | - |  | 2.2 | 1.7 | 16.9 | 81.4 | - | < 0.05 |
| Mean | Total PFSAs | 5.3E-4 | 0.10 | 0.48 | 1.6E-04 | 2.3E-7 | 1.4E-2 | - | - |  | 0.6 | 0.1 | 17.5 | 82.4 | - | < 0.05 |
| Median |  | 2.7E-4 | 0.09 | 0.36 | 1.2E-04 | 2.3E-7 | 1.4E-2 | - | - |  | 0.5 | 0.1 | 20.2 | 79.7 | - | < 0.05 |
| Min |  | n.d. | 0.01 | 0.10 | 3.4E-05 | 6.5E-8 | 4.1E-3 | - | - |  | 0.1 | n.d. | 9.8 | 90.1 | - | < 0.05 |
| Max |  | 1.9E-3 | 0.20 | 1.38 | 4.7E-04 | 4.7E-7 | 2.9E-2 | - | - |  | 1.6 | 0.1 | 12.5 | 87.4 | - | < 0.05 |
| Mean | Total FTOHs | 8.09 | - | 0.71 | 2.4E-04 | - | - | 0.44 | - |  | 9.2 | 87.6 | - | 7.7 | 4.8 | < 0.05 |
| Median |  | 5.22 | - | 0.59 | 2.0E-04 | - | - | 0.26 | - |  | 6.1 | 86.0 | - | 9.7 | 4.3 | < 0.05 |
| Min |  | 1.75 | - | 0.25 | 8.6E-05 | - | - | 0.01 | - |  | 2.0 | 87.0 | - | 12.7 | 0.3 | < 0.05 |
| Max |  | 32.5 | - | 1.81 | 6.1E-04 | - | - | 1.97 | - |  | 36.2 | 89.6 | - | 5.0 | 5.4 | < 0.05 |
| Mean | Total FOSAs/ FOSEs | 0.24 | - | 0.22 | 7.4E-05 | - | - | 0.16 | 8.5E-4 |  | 0.6 | 38.7 | - | 35.7 | 25.4 | 0.15 |
| Median |  | 0.22 | - | 0.16 | 5.3E-05 | - | - | 0.11 | 5.8E-4 |  | 0.5 | 44.9 | - | 33.0 | 22.0 | 0.13 |
| Min |  | 6.0E-2 | - | n.d. | n.d. | - | - | 0.01 | 3.2E-5 |  | 0.1 | 91.1 | - | n.d. | 8.9 | < 0.05 |
| Max |  | 0.38 | - | 0.71 | 2.4E-04 | - | - | 0.53 | 2.9E-3 |  | 1.6 | 23.5 | - | 43.8 | 32.5 | 0.19 |

Table S16. Percent contribution to ΣPFAS, Σ(Ionic PFAS), and Σ(Neutral PFAS) exposure via inhalation, ingestion of water, ingestion of dust/surfaces (and mouthing of clothing for a simulated 2-year old), and dermal uptake with and without estimated dietary exposure. The intake rate of dietary exposure was 250 ng d-1, which was determined by Tittlemier et al. (2007)12 for Canadians over 12 years of age for seven PFCAs and PFOS. As such, percent contribution including diet should be interpreted with caution.

|  | Adult |  |  |  |  |  | 2 - Yr Old |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Exposure ng kg-1 d-1 | % Contribution w/ Diet |  |  |  |  | Total Exposure ng kg-1 d-1 | % Contribution w/ Diet |  |  |  |  |
|  |  | Inhalation | Ingestion - Water | Ingestion – Dust/Surfaces | Dermal | Diet |  | Inhalation | Ingestion - Water | Ingestion – Dust/Surfaces/Cloth | Dermal | Diet |
| Total PFAS | 7.1 | 43.7 | 3.7 | 3.0 | 8.E-02 | 49.6 | 31.3 | 26.9 | 0.9 | 11.9 | 5.8E-03 | 60.3 |
| Total Ionic | 3.9 | 0.1 | 6.7 | 3.6 | 2.E-03 | 89.7 | 21.4 | 3.2E-02 | 1.3 | 10.3 | 3.0E-03 | 88.5 |
| Total Neutral | 3.2 | 97.6 |  | 2.2 | 2.E-01 |  | 10.0 | 84.6 | - | 15.4 | 1.2E-02 | - |
|  | Total Exposure w/o Diet ng kg-1 d-1 | % Contribution w/o Diet |  |  |  |  | Total Exposure w/o Diet ng kg-1 d-1 | % Contribution w/o Diet |  |  |  |  |
|  |  | Inhalation | Ingestion - Water | Ingestion – Dust/Surfaces | Dermal |  |  | Inhalation | Ingestion - Water | Ingestion – Dust/Surfaces/Cloth | Dermal |  |
| Total PFAS | 3.6 | 86.7 | 7.3 | 5.9 | 0.2 |  | 12.4 | 67.9 | 2.2 | 30.0 | 1.5E-02 |  |
| Total Ionic | 0.4 | 0.6 | 64.6 | 34.8 | 1.5E-02 |  | 2.5 | 0.3 | 10.8 | 88.9 | 2.6E-02 |  |
| Total Neutral | 3.2 | 97.6 | - | 2.2 | 0.2 |  | 10.0 | 84.6 | - | 15.4 | 1.2E-02 |  |

Figure S3. Relative contributions to daily exposure to ΣPFAS, Σ(Ionic PFAS), and Σ(Neutral PFAS) for a simulated adult and 2-year-old child from diet, inhalation, ingestion of dust, hand-to-mouth behavior after surface contact, ingestion of tap water, dermal uptake from direct contact with dust and surfaces, as well as air-to-skin uptake for the FOSE/FOSAs only.

Table S17. Estimate of 8:2FTOH biotransformed PFOA contributions to PFOA and Σ(Ionic PFAS) using a biotransformation rate of 0.5%.19

| Adult |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 8:2 FTOH (ng kg-1 d-1) | 8:2 FTOH -> PFOA (ng kg-1 d-1) | PFOA (ng kg-1 d-1) | Σ(Ionic PFAS) (ng kg-1 d-1) | (8:2 FTOH -> PFOA) / PFOA (%) | (8:2 FTOH -> PFOA) / Σ(Ionic PFAS) (%) |
| Inhalation | 1.7 | 8.4E-3 | 1.2E-5 | 2.5E-3 | 68727 | 331 |
| Dermal | 5.2E-6 | 2.6E-08 | 9.6E-7 | 6.0E-5 | 2.7 | 4.3E-2 |
| Ingestion | 1.2E-2 | 6.1E-5 | 6.1E-2 | 4.0E-1 | 1.0E-1 | 1.5E-2 |
| 2 - Yr Old |  |  |  |  |  |  |
|  | 8:2 FTOH (ng kg-1 d-1) | 8:2 FTOH -> PFOA (ng kg-1 d-1) | PFOA (ng kg-1 d-1) | Σ(Ionic PFAS) (ng kg-1 d-1) | (8:2 FTOH -> PFOA) / PFOA (%) | (8:2 FTOH -> PFOA) / Σ(Ionic PFAS) (%) |
| Inhalation | 4.6 | 2.3E-2 | 3.3E-5 | 6.9E-3 | 68727 | 331 |
| Dermal | 5.5E-5 | 2.8E-7 | 2.2E-5 | 6.4E-4 | 1.3 | 4.3E-2 |
| Ingestion | 3.2E-1 | 1.6E-3 | 1.5E-1 | 2.5 | 1.1 | 6.5E-2 |

a)

b)

Figure S4. Contribution of biotransformed 8:2 FTOH to PFOA to estimated PFOA body burden (a) and Σ(Ionic PFAS) body burden (b) for a simulated adult and child.

References:

(1)    Chang, N. Y.; Eichler, C. M. A.; Amparo, D. E.; Zhou, J.; Baumann, K.; Cohen Hubal, E. A.; Surratt, J. D.; Morrison, G. C.; Turpin, B. J. Indoor Air Concentrations of PM2.5 Quartz Fiber Filter-Collected Ionic PFAS and Emissions to Outdoor Air: Findings from the IPA Campaign. Environ. Sci.: Processes Impacts 2024.

(2)    Zhou, J.; Baumann, K.; Mead, R. N.; Skrabal, S. A.; Kieber, R. J.; Avery, G. B.; Shimizu, M.; DeWitt, J. C.; Sun, M.; Vance, S. A.; Bodnar, W.; Zhang, Z.; Collins, L. B.; Surratt, J. D.; Turpin, B. J. PFOS Dominates PFAS Composition in Ambient Fine Particulate Matter (PM2.5) Collected across North Carolina Nearly 20 Years after the End of Its US Production. Environ. Sci. Process. Impacts 2021, 23, 580–587.

(3)    Eichler, C. M. A.; Chang, N. Y.; Cohen Hubal, E. A.; Amparo, D. E.; Zhou, J.; Surratt, J. D.; Morrison, G. C.; Turpin, B. J. Cloth-Air Partitioning of Neutral Per- and Polyfluoroalkyl Substances (PFAS) in North Carolina Homes during the Indoor PFAS Assessment (IPA) Campaign. Environ. Sci. Technol. 2023, 57, 15173–15183.

(4)    Strynar, M.; Medina-Vera, M. Improved Method for Extraction and Analysis of Perfluorinated Compounds (PFCs) from Surface Waters and Well Water by Ultra-High Performance Liquid Chromatography (UPLC)-Tandem Mass Spectrometry (MS/MS); D-EMMD-PHCB-043-SOP-03; US EPA, 2016.

(5)    Gewurtz, S. B.; Bhavsar, S. P.; Crozier, P. W.; Diamond, M. L.; Helm, P. A.; Marvin, C. H.; Reiner, E. J. Perfluoroalkyl Contaminants in Window Film: Indoor/Outdoor, Urban/Rural, and Winter/Summer Contamination and Assessment of Carpet as a Possible Source. Environ. Sci. Technol. 2009, 43, 7317–7323.

(6)    Eichler, C. M. A.; Chang, N. Y.; Amparo, D. E.; Cohen Hubal, E. A.; Surratt, J. D.; Morrison, G. C.; Turpin, B. J. Partitioning of Neutral PFAS in Homes and Release to the Outdoor Environment: Results from the IPA Campaign. Environ. Sci. Technol. 2024.

(7)    U.S. EPA. Exposure Factors Handbook 2011 Edition (Final Report); U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-09/052F, 2011.

(8)    East, A.; Egeghy, P. P.; Hubal, E. A. C.; Slover, R.; Vallero, D. A. Computational Estimates of Daily Aggregate Exposure to PFOA/PFOS from 2011 to 2017 Using a Basic Intake Model. J. Expo. Sci. Environ. Epidemiol. 2021.

(9)    Weschler, C. J.; Nazaroff, W. W. SVOC Exposure Indoors: Fresh Look at Dermal Pathways. Indoor Air 2012, 22, 356–377.

(10)   Kissel, J. C.; Titaley, I. A.; Muensterman, D. J.; Field, J. A. Evaluating Neutral PFAS for Potential Dermal Absorption from the Gas Phase. Environ. Sci. Technol. 2023, 57, 4951–4958.

(11)   Franko, J.; Meade, B. J.; Frasch, H. F.; Barbero, A. M.; Anderson, S. E. Dermal Penetration Potential of Perfluorooctanoic Acid (PFOA) in Human and Mouse Skin. J. Toxicol. Environ. Health Part A 2012, 75, 50–62.

(12)   Tittlemier, S. A.; Pepper, K.; Seymour, C.; Moisey, J.; Bronson, R.; Cao, X.-L.; Dabeka, R. W. Dietary Exposure of Canadians to Perfluorinated Carboxylates and Perfluorooctane Sulfonate via Consumption of Meat, Fish, Fast Foods, and Food Items Prepared in Their Packaging. J. Agric. Food Chem. 2007, 55, 3203–3210.

(13)   Cohen Hubal, E. A.; Egeghy, P. P.; Leovic, K. W.; Akland, G. G. Measuring Potential Dermal Transfer of a Pesticide to Children in a Child Care Center. Environ. Health Perspect. 2006, 114, 264–269.

(14)   Fasano, W. J.; Kennedy, G. L.; Szostek, B.; Farrar, D. G.; Ward, R. J.; Haroun, L.; Hinderliter, P. M. Penetration of Ammonium Perfluorooctanoate through Rat and Human Skin in Vitro. Drug Chem. Toxicol. 2005, 28, 79–90.

(15)   US EPA. Standard Operating Procedures for Residential Pesticide Exposure Assessment; US EPA, 2012.

(16)   Nicas, M.; Best, D. A Study Quantifying the Hand-to-Face Contact Rate and Its Potential Application to Predicting Respiratory Tract Infection. J. Occup. Environ. Hyg. 2008, 5, 347–352.

(17)   Morrison, G.; Shakila, N. V.; Parker, K. Accumulation of Gas-Phase Methamphetamine on Clothing, Toy Fabrics, and Skin Oil. Indoor Air 2015, 25, 405–414.

(18)   Gennings, C.; Hauser, R.; Koch, H. M.; Kortenkamp, A.; Lioy, P. J.; Mirkes, P. E.; Schwetz, B. A. CHRONIC HAZARD ADVISORY PANEL ON PHTHALATES AND PHTHALATE ALTERNATIVES; U.S. Consumer Product Safety Comission: Bethesda, MD, 2014.

(19)   Himmelstein, M. W.; Serex, T. L.; Buck, R. C.; Weinberg, J. T.; Mawn, M. P.; Russell, M. H. 8:2 Fluorotelomer Alcohol: A One-Day Nose-Only Inhalation Toxicokinetic Study in the Sprague-Dawley Rat with Application to Risk Assessment. Toxicology 2012, 291, 122–132.

(20)   Sinakumar, I. T.; Young, D.; Langston, M. Data Processing and Validation; UCD IMPROVE Standard Operating Procedure; Version 6.0.; 351; UC Davis, 2022; pp. 1–12.

(21)   Eriksson, U.; Kärrman, A. World-Wide Indoor Exposure to Polyfluoroalkyl Phosphate Esters (PAPs) and Other PFASs in Household Dust. Environ. Sci. Technol. 2015, 49, 14503–14511.

(22)   Zheng, G.; Boor, B. E.; Schreder, E.; Salamova, A. Indoor Exposure to Per- and Polyfluoroalkyl Substances (PFAS) in the Childcare Environment. Environ. Pollut. 2020, 258, 113714.

(23)   Harrad, S.; Wemken, N.; Drage, D. S.; Abdallah, M. A.-E.; Coggins, A.-M. Perfluoroalkyl Substances in Drinking Water, Indoor Air and Dust from Ireland: Implications for Human Exposure. Environ. Sci. Technol. 2019, 53, 13449–13457.

(24)   Schildroth, S.; Rodgers, K. M.; Strynar, M.; McCord, J.; Poma, G.; Covaci, A.; Dodson, R. E. Per-and Polyfluoroalkyl Substances (PFAS) and Persistent Chemical Mixtures in Dust from U.S. Colleges. Environ. Res. 2022, 206, 112530.

(25)   Hall, S. M.; Patton, S.; Petreas, M.; Zhang, S.; Phillips, A. L.; Hoffman, K.; Stapleton, H. M. Per- and Polyfluoroalkyl Substances in Dust Collected from Residential Homes and Fire Stations in North America. Environ. Sci. Technol. 2020, 54, 14558–14567.

(26)   Winkens, K.; Giovanoulis, G.; Koponen, J.; Vestergren, R.; Berger, U.; Karvonen, A. M.; Pekkanen, J.; Kiviranta, H.; Cousins, I. T. Perfluoroalkyl Acids and Their Precursors in Floor Dust of Children’s Bedrooms - Implications for Indoor Exposure. Environ. Int. 2018, 119, 493–502.

(27)   Fraser, A. J.; Webster, T. F.; Watkins, D. J.; Strynar, M. J.; Kato, K.; Calafat, A. M.; Vieira, V. M.; McClean, M. D. Polyfluorinated Compounds in Dust from Homes, Offices, and Vehicles as Predictors of Concentrations in Office Workers’ Serum. Environ. Int. 2013, 60, 128–136.

(28)   Goosey, E.; Harrad, S. Perfluoroalkyl Compounds in Dust from Asian, Australian, European, and North American Homes and UK Cars, Classrooms, and Offices. Environ. Int. 2011, 37, 86–92.

(29)   Knobeloch, L.; Imm, P.; Anderson, H. Perfluoroalkyl Chemicals in Vacuum Cleaner Dust from 39 Wisconsin Homes. Chemosphere 2012, 88, 779–783.

(30)   De Silva, A. O.; Allard, C. N.; Spencer, C.; Webster, G. M.; Shoeib, M. Phosphorus-Containing Fluorinated Organics: Polyfluoroalkyl Phosphoric Acid Diesters (DiPAPs), Perfluorophosphonates (PFPAs), and Perfluorophosphinates (PFPIAs) in Residential Indoor Dust. Environ. Sci. Technol. 2012, 46, 12575–12582.

(31)   Strynar, M. J.; Lindstrom, A. B. Perfluorinated Compounds in House Dust from Ohio and North Carolina, USA. Environ. Sci. Technol. 2008, 42, 3751–3756.

(32)	Weiss, J. M.; Jones, B.; Koekkoek, J.; Bignert, A.; Lamoree, M. H. Per- and Polyfluoroalkyl Substances (PFASs) in Swedish Household Dust and Exposure of Pet Cats. Environ. Sci. Pollut. Res. Int. 2021, 28, 39001–39013.