Supporting Material

Measurement of Urinary Benzo[a]pyrene Tetrols and Their Relationship to Other PAH Metabolites and Cotinine in Humans

Donald C. Hilton*, Debra A. Trinidad, Kendra Hubbard, Zheng Li, Andreas Sjödin

Centers for Disease Control and Prevention, 4770 Buford Highway, Atlanta, GA 30341

*Corresponding Author

Donald C. Hilton

National Center for Environmental Health

Centers for Disease Control and Prevention

4770 Buford Highway NE, Atlanta, GA 30341, USA

E-mail: DHilton@cdc.gov

Tel. +1 770-488-4207 Fax: +1 770-488-0142

Table S1 Demographics of Study Participants.

|  |  | Smoker |  | Non-Smoker a |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Male | Female | Male | Female |
| Caucasian | N | 18 | 10 | 8 | 14 |
|  | Age Range (years) | 18-53 | 21-63 | 19-47 | 19-65 |
|  | Median Age (years) | 22.5 | 23.5 | 25.5 | 26 |
| Asian | N | -- | -- | -- | 3 |
|  | Age Range (years) |  |  |  | 22-75 |
|  | Median Age (years) |  |  |  | 47 |
| Hispanic | N | -- | -- |  | 1 |
|  | Age Range (years) |  |  |  | 59 |
|  | Median Age (years) |  |  |  | 59 |
| Black | N | 2 | -- | -- | -- |
|  | Age Range (years) | 23-23 |  |  |  |
|  | Median Age (years) | 23 |  |  |  |
| ALL | N | 20 | 10 | 8 | 18 |
|  | Age Range (years) | 18-53 | 21-63 | 19-47 | 19-75 |
|  | Median Age (years) | 23 | 23.5 | 25.5 | 34.5 |

a Four nonsmoking subjects were excluded from the data analysis because cotinine in urine indicated noncompliance (cotinine > 100 ng/mL).

Table S2, Instrumental Conditions used for GC/MS analysis.

| Trace GC |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Oven |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ramp Rate |  | Final Temperature |  | Hold Time |  |  |  |  |
|  |  |  | -- |  | 70 °C |  | 3.5 min |  |  |  |  |
|  |  |  | 30 °C/min |  | 310 °C |  | 6.0 min |  |  |  |  |
|  | PTV |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Base Temperature: |  |  | 70 °C |  |  |  |  |  |
|  |  |  | Split Flow: |  |  | 20 ml/min |  |  |  |  |  |
|  |  |  | Splitless Time: |  |  | 2.00 min |  |  |  |  |  |
|  |  |  | Solvent Valve Temperature: |  |  | 140 °C |  |  |  |  |  |
|  |  |  |  |  | Pressure (kPa) | Time (min) |  | Rate (°C/min) | Temperature °C |  | Flow (mL/min) |
|  |  |  | Injection |  | 50 | .3 |  |  | 70 |  | 100 (vent) |
|  |  |  | Evaporation |  | 92 | 1.3 |  | 0.1 | 70 |  |  |
|  |  |  | Transfer |  | 210 | 1.5 |  | 10.0 | 310 |  |  |
|  |  |  | Clean |  |  | 14.0 |  | 10.0 | 310 |  | 100 |
|  |  | Column Flow: |  |  |  | 0.90 mL/min, Constant flow |  |  |  |  |  |
|  |  | MS Transfer Line |  |  |  | 270 °C |  |  |  |  |  |
| TriPlus Autosampler |  |  |  |  |  |  |  |  |  |  |  |
|  | Sample Volume |  |  |  |  | 10.0 µL |  |  |  |  |  |
|  | Air Volume |  |  |  |  | 1.0 µL |  |  |  |  |  |
|  | Pre-injection dwell time |  |  |  |  | 0.5 s |  |  |  |  |  |
|  | Post-injection dwell time |  |  |  |  | 1.0 s |  |  |  |  |  |
|  | Viscosity |  |  |  |  | Viscous |  |  |  |  |  |
|  | Pre- and Post-injection washes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Solvents |  |  | A,B,-,- (A= EtOAc, B=Toluene) |  |  |  |  |  |
|  |  |  | Solvent Cycles |  |  | 5 |  |  |  |  |  |
|  |  |  | Solvent Volume |  |  | 15 µL |  |  |  |  |  |
| TSQ MS |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ionization |  |  |  | NCI |  |  |  |  |  |
|  |  | CI Gas (Ammonia) flow |  |  |  | 2.2 mL/Min |  |  |  |  |  |
|  |  | Source Temperature |  |  |  | 200 °C |  |  |  |  |  |
|  |  | Emission Current |  |  |  | 50 µA |  |  |  |  |  |
|  |  | Electron Energy |  |  |  | -100 eV |  |  |  |  |  |
|  |  | Q2 Gas (Argon)Pressure |  |  |  | 1.5 |  |  |  |  |  |
|  |  | Chrom Filter |  |  |  | 1.5 |  |  |  |  |  |
|  |  | MRM Transitions |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Compounds Monitored |  | Transition |  |  |  | Collision Energy |  |
|  |  |  |  | 13C12 PCB 209 (recovery standard) |  | 509.50 |  |  |  | 0 |  |
|  |  |  |  | B[a]P tetrahydro tetrols |  | 446.00 → 255.20 |  |  |  | 18 |  |
|  |  |  |  | B[a]P tetrahydro tetrols |  | 446.00a → 284.20 |  |  |  | 18 |  |
|  |  |  |  | 13C6 B[a]P tetrahydro tetrols |  | 452.50 → 261.20 |  |  |  | 18 |  |
|  |  |  |  | 13C6 B[a]P tetrahydro tetrols |  | 452.50a → 290.20 |  |  |  | 18 |  |

a Second transition from a given mass is set at 0.001 mass unit higher to work around an instrument setup limitation.

Table S3, Concentrations of PAH Metabolites (pg/mL) after exposure to a barbequed meal.

|  | Time to Urine Collection after Exposure (Hr.) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 | 7 | 17.5 | 20.5 | 21.5 |
| Analyte |  |  |  |  |  |
| 1-Hydroxypyrene | 70 | 2773 | 1380 | 382 | 38 |
| 2-Hydroxynaphthalene | 3869 | 87525 | 15143 | 7116 | 772 |
| 3-Hydroxyfluorene | 142 | 3274 | 1579 | 719 | 83 |
| BPT I-1 | <LOD | 0.98 | 0.91 | 0.35 | <LOD |
| BPT II-1 | <LOD | 0.97 | 0.65 | 0.15 | <LOD |

Table S4, Comparison of PAH biomarker results with data from the 2011-2012 NHANES.(CDC, 2015)

|  | Current Study |  | 2011-2012 NHANES |  |
| --- | --- | --- | --- | --- |
|  | Nonsmoker | Smoker | Nonsmoker | Smoker |
| Compound | GM (pg/mL) | GM (pg/mL) | GM (pg/mL) | GM (pg/mL) |
| 1-Hydroxynaphthalene | 793 | 5,060 | 1,190 | 9,590 |
| 2-Hydroxynaphthalene | 2,370 | 14,100 | 3,170 | 12,400 |
| 2-Hydroxyfluorene | 183 | 717 | 164 | 1160 |
| 3-Hydroxyfluorene | 53.5 | 351 | 58 | 605 |
| 9-Hydroxyfluorene | 261 | 513 | 207 | 609 |
| 1-Hydroxyphenanthrene | 131 | 201 | 115 | 198 |
| 2-Hydroxyphenanthrene | 78.5 | 126 | 54 | 113 |
| 3-Hydroxyphenanthrene | 69.7 | 159 | 49 | 140 |
| 4-Hydroxyphenanthrene | 25.1 | 41.1 | 18 | 39 |
| 1-Hydroxypyrene | 100 | 223 | 83 | 243 |

REFERENCES

CDC. Fourth Report on Human Exposure to Environmental Chemicals, Updated Tables (February, 2015), Centers for Disease Control and Prevention, Atlanta, 2015, https://www.cdc.gov/exposurereport/ accessed Jan 23, 2017.